Name: 	____________________________

EID:   	____________________________

Semester: Spring 2026

Course: ECE445L

A) Objectives:

- Review ECE319K, TM4C123, and Keil (or CCS12.8), learn how to use the lab power supply, DVM and the analog scope.

B) Hardware Design Deliverables:

- Deliverable 1: Using KiCad, draw the switch and LED circuits showing the connections to the microcontroller.

C) Software Design Deliverables:

- I have pushed my code to Git Hub for grading (Check box if true).

☒

D) Measurement Data:

- Deliverable 1: LED current versus voltage for 4 voltages around 2V (lab power supply).

1.8V :38 mA, 9 mA

1.9V : 46 mA, 17 mA

2.0V : 56 mA, 26 mA

2.1V : 66 mA, 37 mA

- Deliverable 2 LED current and voltage measured with DVM. Calculate LED power.

Voltage : 1.787 V  220 ohm resistor 1.85 V 1k ohm resistor

Current : 4.63 mA ohm resistor 1.43 mA 1k ohm resistor

Power : 8.27 mW ohm resistor 2.65 mW 1k ohm resistor

- Deliverable 3: Analog scope measurements showing the switch bounce. Determine the longest time the switch bounces.

We couldn’t capture any bounce after trying to bounce the switch for a long time, so the TA’s told us to just draw what bounce would look like.

D) Analysis and Discussion (1 or 2 sentences each).  In particular, answer these questions:

1) Compare your LED measured data with the LED data sheet. How are they similar and how are they different?

The measured LED data follows the same overall trend as the datasheet, where the forward current increases as the forward voltage increases. For example, at 2.0 V the datasheet lists a forward current of about 20 mA, while the measured current was approximately 26 mA; this difference is likely due to normal device-to-device variation and differences between the tested LED and the datasheet reference.

2) How did you set the scope mode and trigger to collect the bounce trace?

The oscilloscope trigger was set to edge mode so that it would activate on the button transition. Single acquisition mode was used to capture the first switching event and hold the waveform on the screen for analysis.

3) Explain how you removed the switch bounce. 
What would have been other approaches to remove bounce? Switch bounce was removed by adding a 10 ms delay after each detected button press so that rapid transitions caused by mechanical bouncing were ignored. Another approach would be to use a SysTick timer to sample and run the FSM at fixed 10 ms intervals instead of using delays in the main loop.

4) Explain your software approach to “toggle the LED every third touch”. What are the pros and cons for implementing such a simple software problem with an FSM abstraction?

An FSM was used to count complete press-and-release cycles and toggle the LED only after the required number of valid touches, preventing bounce from being counted as extra presses. This approach is well-structured and robust, but it requires more code and complexity than a simple edge-detection counter.